| DESCRIPTION |  |
| --- | --- |
| Light weight, portable lockstitch machine with reversible drop feed and electric drive | The motor is mounted on machine bed. Wiring to motor and light is concealed in machine arm and bed. The machine is equipped with a foot controller. (Note: Machine 221K4 is also equipped with R. F. suppressers and grounded wiring.) |
| Graduated stitch length indicator. | Light and light shade attached to front of machine arm with control switch located on bed. (Machine 221K4 has light switch built into the light socket.) |
| Numerically graduated thread tension device. | Machine base 10-7/64 inches long, 4.1/4 inches wide. Space at right of needle, 5 inches. |
| Rotary hook on horizontal axis, makes two revolutions per stitch. | Presser bar light: 5/16 inch. Feed dog height: .040 inch above throat plate |
| Machine 221K7 is equipped with belt linkage between arm and hook shafts. Other varieties are equipped with an upright arm shaft and gears. | Maximum stitch length: 6 stitches per inch. |
| Hinged bed extension | Speed range: 900 to 1400 r.p.m. |
| Manually controlled bobbin winder attached to belt guard. |  |
| Needle, Cat. 2020 (15 x 1) threaded from right to left. |  |