REMOVALS AND REPLACEMENTS
MOTOR

| CAUTION: Remove plug from electric outlet before removing any electrical parts from machine WITH THREE-PIN TERMINAL REMOVAL: 1. Remove bottom cover. 2. Loosen motor mounting screw, Fig. 50, and remove motor belt. 3. Remove three-pin terminal fastening screw, Fig. 50. 4. From reverse side of terminal, disconnect black motor lead from terminal pin 2 and red lead from terminal pin 3, as shown in Fig. 51. 5. Loosen motor leads clamping bracket screw in base of machine. 6. Remove motor mounting screw, Fig. 50, and remove motor, drawing motor leads out through opening in machine frame. REPLACEMENT: 1. Replace motor and wiring in reverse order of its removal. 2. Replace motor belt and adjust belt tension. WITHOUT THREE-PIN TERMINAL REMOVAL: 1. remove bottom cover. 2. Loosen motor mounting screw, Fig. 50, and remove motor belt. 3. From bottom of machine, disconnect motor leads from controller and power leads as shown in Fig. 52. 4. Mark or tag all leads to aid in reconnection during replacement. 5. Remove motor mounting screw, Fig. 50, and remove motor, drawing motor leads out through opening in machine frame. REPLACEMENT: 1. Replace motor and wiring in its reverse order of its removal. 2. Replace motor belt and adjust belt tension. |  |
| --- | --- |