TO SET NEEDLE BAR HEIGHT

| USING TIMING MARK CHECK: Turn hand wheel over toward operator until needle bar is at its lowest point. At this position, upper timing mark, Fig. 14, should be just visible at lower edge of needle bar bushing. SETTING: Loosen clamp screw, Fig. 14, and raise or lower needle bar as required. Make certain needle bar is correctly turned with long groove in needle facing toward right of machine. Then tighten clamp screw. WITHOUT USING TIMING MARK If there is any belief that the needle bar bushing has been disturbed, it will be impossible to set the needle bar by use of the timing marks. CHECK: Remove bobbin case and bobbin. With needle threaded, hold thread end and turn hand wheel over toward operator until needle reaches its lowest point and rises approximately 1/10 inch. At this point the needle thread loop should be fully formed, Fig. 15, and ready for entrance of hook point directly above needle eye. SETTING: Loosen clamp screw, Fig. 14, and raise or lower needle bar as required. Make certain long groove of needle is facing toward right of machine and tighten clamp screw. Then set the needle bar bushing. TO SET NEEDLE BAR BUSHING CHECK:With needle bar set of correct height, turn hand wheel over toward operator until needle bar is at its lowest point. In this position the upper timing mark on the needle bar should be at bottom edge of the bushing. SETTING:Loosen needle bar bushing set screw, Fig. 14, and raise or lower bushing as required. Then tighten set screw. |  |
| --- | --- |